Nikon Ti startup guide
last updated 16 June 2021

The Nikon Ti has independent light paths depending on the type of imaging.

The first part of the instructions is common for all imaging modes.

Basic Startup

- One power strip “Main” on top of computer turns on most of the components.

- If computer already on, log out and log in (use lower left Windows icon).

- If computer off, turn it on, button on top.

- Run Nikon Elements software.

- Choose one camera.
“Andor Neo/Zyla" is the most common camera for standard imaging.”

- Macro > Run Macro From File

C:\Program Files\NIS-Elements\Macros\delete all settings.mac

or

F:\Your_Directory\delete all settings.mac

- Calibration > Optical Configurations (Ctrl N)

Import Optical configurations

F:\Your_Directory\last_file_that_worked_well.xml

When using ND Acquisition to take pictures, bit depth, gain, and readout speed should be the same for all channels.

Shutdown

- Export a copy of your latest working version to your directory. 
Calibration > Optical Configurations (Ctrl N)

Export Optical configurations

- Lower focus manually so lens is not near the stage

- Exit Nikon Elements.

- Copy files to research server.

- If oil lens, clean oil by swiping with taut lens tissue.  Single swipe, then clean part of tissue.  Do not press down on lens with tissue!

- Clean microscope area.  No kimwipes, tips, etc left on tables.

- Log out or turn off computer.

- Main power switch off.